# Village Council Walk Around 7/29/15		Tanner, Palladino, DeBisschop

Aster:  Next year stone should be trenched along the broken pavement so that water runs into the culvert.  The stairs at 10 Aster need several bricks repaired.  The entire set up stairs will need work in the next 5-7 years.  Once the trees are removed along Ivy-Aster, yearly brush maintenance should be budgeted.

In the longer term Aster will need to be repaved.  Drives are part of the road, so they should be done at same time.  Mill and resurface or complete dig up to be determined.  A some point it may be wise to consider a snow driveway in between 14 and 11.  It would provide extra parking and it would be a place to push the snow into the ravine.

Ivy:  The WO has been placed for clearing remaining area behind KWN to corner behind 23 Ivy.  The irrigation control valve boxes have to be located for Ivy and Aster. Wire trace this fall.  Irrigation behind 21 and 23 will be fitted with ball valves so as to be able to push more pressure to front yards.  There is probably still a root restriction between 21 and 23 in the front yard.  Hosta or tall grasses should be planted along edge of 23 drive in front of rip-rap.  The drainage situation between 22 and 24 should be corrected in September of 2014 by removing the three trees, root clearing the area, digging a trench from the driveway to where the three trees are removed.  Dig a Y or connection trench from the lower corner of the foundation into the trench from the drive.  Fill both with stone.  Plant the three bushes along the foundation.  Put a bank of stone up against step to direct water away into the ravine.

Pot holes: patch or cut out and resurface?

Conservancy should clear large trees behind Ivy-Aster.

KWN:

Two pines at 36 KWN will come down this fall, 2015.  The maple at 39 KWN will have to be watched.  It is weak.  Large pines behind 39 and next to 31 will be an issue in 3-5 years.

Sagewood:  entrance should be widened when road is repaved (or before.)

Heather:  left hand side as you enter may need to be repaved as it is sunken.

Primrose:  Pot holes: patch or cut out and resurface?  Widen the curve in front of 136.  Drives in good shape. First half of road is not in good shape. Second half is in good shape.  Planting barrier behind 133 and MG can still be saved by removal of some of interior bushes.

Morning Glory: Pot holes: patch or cut out and resurface?  Will involve raising water main covers.  Widen the curve in front of 142.  Irrigation behind 144 and 146 can be extended from corner of 144.  Yearly clearing maintenance of area from 146 to KWS exit.  Ongoing tree thinning.

KWS: Pot hole near entrance: patch or cut out and resurface? Wire trace irrigation control valve boxes this fall.  Irrigation: redistribute heads from strong area in back of 105 and 107 to front and sides of 105 and 107.  Cut out and resurface  man hole cover on KWS at MG.   Ongoing tree thinning from KWS exit to KW entrance.  Crack seal drive 103.

On the Green and Hawthorne: trim the large trees between 5 OTG and 527/523 Hawthorne.

On the Green and Mercer:  the large barrier in back of 4 OTG is probably too large to do anything with other than continue to trim.

Hawthorne: overgrown yew in back of 511. Leave other plants.  Same with yew to side of 527..  Both are purely aesthetic issues.

In General: a minimum of $10,000 should be budgeted for tree work every year and it should be spent!  Fences should be power washed every three years.  Brush lines should be cleared from Lockview to Ivy/Aster to WM once every year.  Thinning should be considered from Lockview to KWS exit.  Pot holes should be done in SPRING!  Catch basins should be cleaned every 2-3 years or whenever rest of WW basins are scheduled to be cleaned.  Mail box plan should be implemented on the Knoll first and as soon as is possible.  The One Way, Do Not Enter, Exit Only, and No Parking signs should be mounted on 4x4 PT posts sunk at least 3’ into the ground.

Finish in 2015:

1. the designated walkways replacements and partial repairs

2. the clearing from KWN to corner behind Ivy

3. the two big pines at 36 KWN

4. the drainage at 22/24 Ivy

5. the minor rip out dead plants and replace a few already in progress

6. the Knoll and Lower Village fall big shrub 50 % cut backs already ordered

7. the catch basins, pot holes, and man hole cover

8. straighten up new plantings on Lockview

9 irrigation control box locating

10 white fence repair at Sagewood

11. wooden turn around barrier at Aster

2016 Rebuild walkways already selected. See spreadsheet.